# GEO® | Double blade louvre – mounted with integrated carrier

Double blade louvre consisting of a substructure with integrated carrier and prefabricated panel elements.

The panel element is suspended rigidly from the mounting level [e.g. bare ceiling] with a suspension hanger by means of M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964].

The distance between the carriers depends on the specified module dimension – according to the element size length.

The panel elements are mounted at the module spacing and consist of cells with U-shaped sharp-edged blades. The panel element consists of longitudinal and cross blades. The blades of the short side are connected form-fitting to the integrated carrier by means of the formed hooks. The individual panels are self-adjusting and can be removed individually. The transverse mounted on the panel ensures the correct alignment of the panel elements.

Typically, the edge connection is carried out by means of an L-shaped perimeter trim.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – GEO® double blade open cell ceiling Panel size [LxW]:  1200x600mm  1250x625mm Blade width/blade height:  4/42mm  9/40mm  15/37mm Cell size: ............... mm [50-200mm; precise dimensions on request] Colour:  White  Natural anodized  RAL 9006  RAL 7012 Other colours on request Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | L-shaped perimeter trims Height: 40mm Width: 20mm Length: 2000mm Material: Aluminium, pre-coated Colour:  White  Black  RAL 9006  Natural anodized Unit price: ............… €/rm Total price: ............… € |