WoodTrends Acoustical Wood Ceiling System

WoodGrill Specification

SECTION 09570 – WOOD CEILINGS / WALLS

# PART 1 – GENERAL

- GENERAL

- WoodTrends WoodGrill Acoustical Wood Ceiling Systems covered by this specification shall be the products of Sound Seal, 50 H.P. Almgren Drive, Agawam, MA 01001; Phone (800) 569-1294 (413) 789-1770;  Fax (413) 789-4444

1.01	SCOPE

- Sound Seal shall furnish all WoodTrends WoodGrill acoustical wood panel assemblies necessary to complete installation by contractor in accordance with plans and specification.

1.02	RELATED WORK BY OTHERS

A.	Substrate walls systems and structure and miscellaneous attachment devices other than the WoodGrill clips or those specified herein, are not included.

1.03	QUALITY ASSURANCE

- Installer Qualification: The installer shall be a firm with a minimum of two (2) years of successful experience in installation of products with similar requirements.

- Fire Performance Characteristics: WoodTrends veneered acoustical wood panel systems should be constructed using fire retardant components meeting or exceeding the requirements of the ASTM E-84 Class A

- Flame Spread: 25 or less

- Smoke Developed: 450 or less

- Noise Reduction Coefficient: The WoodTrends WoodGrill systems will have a minimum NRC value of ____  , with a (perforation, slot) pattern measuring ____ , when tested in accordance with ASTM C-423.

1.04	PROJECT CONDITIONS

Installation shall be done only when the temperature and humidity closely approximate the interior conditions that will exist when the building is occupied. The heating and cooling systems shall be operating before, during, and after installation, with the humidity of the interior spaces maintained between min. 45% - max. 65%.

Prior to installation panels shall be allowed to stabilize on site 72 hours prior to installation, the site must be free of wet and dusty trades and the climate conditions stabilize to normal operational levels.

1.05	SUBMITTALS

- Submit shop drawings prepared by the manufacturer showing all necessary details and dimension requirements which will subsequently be field verified and revised as required by the Architect.

B.	Samples: Submit (3sets) 12” by full width sample panels of each type of product and finish as specified. Product shall be original production in veneer finish specified for final use.

C.	Certification: Submit to the owner a certificate of compliance to specified acoustical and fire performance criteria signed by an officer of the panel manufacturer and attach independent laboratory test results for each product used, showing that the products supplied as components meet or exceed the specified requirements.

D.	Single Source: All custom acoustical wall and ceiling assemblies shall be purchased from a single manufacturer.

1.06	DELIVERY, STORAGE, AND HANDLING

- Grill assemblies shall be delivered to the project site in original, unopened packages.

B.	Prior to grill installation the site must be free of all wet and dusty trades and the climatic conditions stabilized to normal operational levels.

C.	The panels shall be stored in a fully enclosed space. For a minimum of seventy-two (72) hours immediately prior to installation, store the panels in the room in which they will be installed. The temperature and humidity of the room shall closely approximate those conditions that will exist when the building is occupied. Store the panels off the floor.

D.	Care in handling MUST be exercised to avoid damage.

1.07	WARRANTIES

A.	Manufacturer: All materials supplied by the manufacturer shall be guaranteed against manufacturing defects for one year.

B.	Contractor: All work shall be guaranteed for one year from final acceptance of completed work.

## PART 2 – PRODUCTS

2.01	WOODTRENDS ACOUSTIC PANELS

- Furnish and deliver prefabricated WoodGrill panels as described in this section for installation in areas as shown on drawings meeting or exceeding the following requirements:

Description: WoodTrends WoodGrill panels, as manufactured by:

Sound Seal 		Tel: (800) 569-1294

P.O Box 545		Tel: (413) 789-1770

50 H.P Almgren Dr.	Fax: (413) 789-4444

Agawam, MA 01001	E-mail: dgilbert@soundseal.com

2.02	Grills

- Substrate of  fire-retardant 16 mm thick MDF with 0.6mm veneer

- Solid Wood Blade with Class A Rated Finish

- Blade Sizes – 16mm wide x 1-1/4”, 1-3/4”, 2-1/4” & 2-3/4” High –

Custom Heights Available

- Available with 4, 6 or 8 Blades per Panel

- Clip detail

- Proprietary clip engages heavy-duty 15/16” grid

- Acoustics achieved by means of (Sound Quality Black Theater board), or

(Soundtex Acoustic Textile)

2.03	PANEL FINISHES

- Wood Species to be:

- Solid Wood Blade, or:

- First Quality select veneer from manufacturer’s standard selection of 24 real wood species.

- Additional 40 species available with special veneers staining and lacquering to match architects samples.

- Lacquer

- Two coats UV- polyacrylate

- Class A Rated on Solid Wood PAnels

- Stain: optional

2.04	SUPPORT SYSTEMS

- Standard blade arrangements shall be installed by means of grill clip on heavy-duty 15/16” T-grid. The panels are to be supported by grill clip at a distance of no more than 24”.

2.05	ACOUSTICAL PERFORMANCE

Sound Absorption: Per ASTM C – 423

Sound Absorption Coefficient per Octave Band Center Frequency (Hz)

125	250	500	1000	2000	4000		NRC

See Acoustic Test Data page for selections

PART 3 – EXECUTION

3.01	PREPARTION

- Layout:  If required, prior to manufacture, field dimensions shall be taken to determine panel sizes. The contractor shall measure applicable areas prior to installation to confirm application and location of panels supports, in accordance with installation instructions.

- Coordination: The contractor shall coordinate with other trades the location of devices which will penetrate the panels or interfere with the installation. Recessed or surface devices located within the panels are to be located and cut in the field.

3.02	INSTALLATION

- General: Install materials in accordance with manufacturer’s printed instructions and approved shop drawings. The installation shall comply with applicable regulations and industry standards.

- Provide shimming and adjustments as required to maintain consistent alignment of joints and of finished blade faces.

3.03	ADJUSTMENT, CLEANING, AND REPAIR

- The contractor shall make final adjustment to level or contours.

- Upon completion of installation, all WoodGrill panels shall be cleaned free of dirt, dust, grease, oils, and fingerprints. Wood surfaces shall be wiped with furniture polish to enhance the surface finish.

- All work, which cannot be successfully cleaned or repaired, shall be removed and replaced.

3.04	INSPECTION

- Upon completion of installation, the owner’s representative shall inspect all finished surfaces to ensure that work has been performed in a manner satisfactory to the owner. Any deficiencies in the installed panels shall be corrected by the contractor at no additional cost to the owner, or to the manufacturer.